- Configure Raspberry Pi to enable I2C interface

- Go to menu: Preferences -> Control Centre, and open Control Centre GUI.

- Find Interfaces tab and enable I2C interface

- Detect I2C devices

- Run command and check i2c devices at address 0x20 and 0x68

i2cdetect -y 1

- The address 0x20 is the controller board with buttons and lights

- The address 0x68 is RTC board

- Install WiringPi library

- Clone WiringPi library

git clone https://github.com/WiringPi/WiringPi.git

- Go to WiringPi folder, build library and install

./build

- Clone i2c demo, make and run

git clone https://github.com/KevinZ-FI/adk-button-and-led-light-demo.git

make

./adk-button-led-light-demo